GCC Driver Installation & Application Notes

Step 1.  Insert the driver diskette to Drive A:

Step 2.  Run the procedure of Set Up A New Printer in Windows

Important Notes !!

Note 1.  Due to the limitation of MS Windows, the maximum acceptable media length is 4000mm.

Note 2.  To control the cutting plotter correctly, the paper size in the GCC driver has to match the page setup in the application program.

Step 1.	The following example is given in Corel Draw 7.  In Corel Draw, you can set up a desirable page size.

Step 2.  Select Property in the Printer Setup dialogue box.

Step 3.	Make sure the media size in the driver matching the Page Setup in application program.  Landscape is recommended as the media orientation.

Finally and very importantly, before you output file to the cutting plotter, you have to set the line width to the thinnest value in whichever application software your are using.  In Corel Draw, for instance, the default line width is 0.076mm; you must change all lines width to 0.001mm in order to have a correct output.

GCC Customer Service Dept.

June, 1998